/** \page executable Building an executable

Making the actual programs

\section exe_intro What is in a DUECA executable

In the end, we want to run a simulation, in real-time on a slew of

different computers. What is needed is a program that does the

simulation. Now the previous sections explained how to set up a data

flow diagram for a simulation, and how to implement modules in

DUECA/DUSIME. What remains to be done is the creation of some sort of

"main program", or in fact several main programs, one for each node,

and combining all modules into executables for each of these nodes.

In DUECA, these two steps are needed, but they are done in reverse

order. First we "throw together" all modules and the code from the

DUECA library, and in a second step the "main" program is made. This

main routine is not in C++, but in scheme, and it is the script that

describes the construction of the simulation model.

\section exe_modules Organising modules

In a first step, it helps to get your modules together. Consider

the MMSLabStick module of the example. This module depends on some IO

code to communicate with the A/D and D/A conversion cards that drive

the stick hardware, it uses a class that is not a module

(the PassiveManipulator class, not further elaborated here) that does

the model calculations, and all this is combined in the MMSLabStick

module, which does the interfacing with DUECA, calls the stick IO and

calls the PassiveManipulator class to do the simulation.

No I recommend (but you may disagree here, and provide your own

solution), to keep all code for a module in a single directory, and,

in an intermediate step, link an object file with all this code. A

template \ref module_make "Makefile" for making a module object file

can be found in the DUECA distribution. Now, when everything works,

typing "make" in that directory gives you a file "module.o" with all

code for the module in it.

\section exe_program Organising an application

The next step is combining all your module.o files and the

DUECA/DUSIME library code into an executable file. My proposal is

again to create a directory for the program, and make the directories

for the modules all sub-directories for this directory. The template

\ref exe_make "Makefile" for making a DUECA executable can be found in

the DUECA distribution. Now, when everything works, typing "make" in

that directory gives you an executable "dueca_run.x".

The peculiar thing about DUECA is that the executable is just a

collection of modules. Running the executable, without providing

additional data, cannot produce sensible output. Two configuration

scripts, described in the Section

\ref configure "on configuration files"

determine the behaviour of your executable. The first one

"dueca.cnf" describes how the executable in a node should interact

with the executables in other nodes, and the second one, "dueca.mod"

describes the composition of the simulation.

*/